SECTION 07 71 23
MANUFACTURED GUTTERS AND DOWNSPOUTS
PART 1 - GENERAL

1.01
RELATED DOCUMENTS
A. The provisions included under Division 1, General Requirements, are included as part of this section as though bound herein.

1.02
SUMMARY

A. Provide labor, material, and equipment necessary for furnishing a complete installation of commercial gutter system.

B.
Related Work Specified Elsewhere

1.
Division 5 Sections for support framing.

2.
Division 6 Sections for nailers and support framing.
3.
Division 7 Sections for related roofing materials.
4.
Division 15 Sections for related plumbing.
1.03
SUBMITTALS

A. Product Data: Each type of product specified. Submit manufacturer's detailed technical product data, installation instructions and recommendations, dimensions of individual components, profiles, and finishes

B. Shop Drawings: Show fabrication and installation of commercial gutter system including fully dimensioned roof plans, expansion joint locations, sections and details of components and other related trims.

C. Finish & Color Selection: Furnish manufacturer's technical data for specified finish and color chart showing full range of colors available.

1.04
QUALITY ASSURANCE
A. Where pre-engineered manufactured products are specified, other field fabricated or shop/field fabricated substitutions will not be accepted. However, where shop/field fabrications are indicated pre-engineered systems will be considered with Architect approval.

B. Obtain all components and related accessories from one single source manufacturer.

C. Follow manufacturer's printed instructions for installing commercial gutter system. Follow primary roofing manufacturer's printed instructions for installing associated roof material for flashing gutter system to roof.

1.05
DELIVERY, STORAGE & HANDLING
A. All products delivered shall be stored in a clean dry location prior to installation.

B. Products furnished with strippable protective masking shall not be exposed to direct sunlight for more than 30 minutes without removing masking.

C. Do not install finished materials with scars or abrasions.
1.06
PROJECT CONDITIONS
A. Coordinate work of this Section with adjoining work for proper sequencing to ensure protection from inclement weather and to protect materials and their finish against damage.

B. Do not install commercial gutter system during inclement weather. When installing in cold climates, warm adhesives, caulks, and primers to at least 50 degrees Fahrenheit prior to application.

PART 2 PRODUCTS

2.01
MANUFACTURERS
A. Provide commercial gutter system, accessories, and drainware as manufactured by Perimeter Systems, a division  of Southern Aluminum Finishing Company, Inc. 8370 East Hwy 78, Villa Rica, GA  30180, (800) 334-9823, Online at http://www.saf.com/persys
2.02 TYPE

A. Provide Perimeter Systems' Designer Series Commercial Gutter System "Colonial Profile Type 2 Drain", 8" size, Model Number DSC-8D2.
2.03
MATERIALS & FABRICATION
A.  Concealed Gutter Liner shall be manufactured from 0.040" mill finished aluminum in 10'-0" lengths. Liner shall be:
1.
Factory notched to receive brackets and straps.

2.
Manufactured with 1" telescoping and notched end laps.

3.
Factory punched with fastening holes elongated to allow for thermal movement.

A. Support Bracket shall be manufactured from 0.125" x 1.00" aluminum, heliarc welded construction, factory punched for fasteners.

B. Interior Straps shall be manufactured from 0.125" x 1.00" aluminum.

C. Colonial Profile, Snap-On fascia, shall be manufactured from 0.040" aluminum, 10'-0" lengths. Fascias shall be press formed with radius dies on a CNC Press to provide repeated true and accurate profiles.

D. Fascia Splices shall be manufactured from 0.040" aluminum, 6" lengths, formed to fit the inside of the Snap-On fascia.

2.04
ACCESSORIES
A. Mitered Corners: Provide factory mitered corners for both fascia and liner. Fascia profiles shall be precision saw cut, heliarc tack welded to produce a picture frame joint. Concealed liner miter shall be precision saw cut with a continuous heliarc weld watertight joint.

B. Sculptured End Caps: Provide factory mitered end caps for fascias. Fascia profiles shall be precision saw cut, heliarc tack welded to produce a picture frame joint.

C. Liner End Caps: Provide mill finished aluminum liner end caps at all Fascia End Caps and wall abutments.

D. Liner Expansion Joint: Provide manufacturer's elastomeric expansion joints at 40' intervals or as shown on shop drawings.
E. Sealant: Shall be polyurethane or silicon based water-proofing type, compatible with aluminum gutter, downspout, and abutting dissimilar materials for intended application.
2.05
DRAINWARE

A. General: Drainage for this commercial grade gutter system is to be accomplished with concealed internal drains furnished and installed by this contractor. Connection and piping shall be furnished by other trades, however, it is this contractor’s responsibility to coordinate the installation.

B. Provide internal drain receiver manufactured from 0.040" aluminum, factory punched to receive concealed drain and flashing clamp at all drain locations.

C. Provide concealed drain, Type 2, with 4" threaded outlet. Drain shall be manufactured from cast iron and furnished with flashing clamp and removable leaf grate.

2.06
FINISHES
A. General: Apply coatings to exposed aluminum components after fabrication for maximum coating performance and to prevent crazing, abrasion, and damage to finished surfaces.

B. Pretreatment: Aluminum components shall be pretreated with solutions to remove organic and inorganic surface soils, remove residual oxides, followed by a chrome phosphate conversion coating to which organic coatings will firmly adhere.

C. Coating Type: High Performance Coating, two-coat, shop applied, 70% Polyvinylidene Fluoride (PVDF) coating based on Elf Arkema Chemicals, Inc. Kynar 500 or Ausimont U.S.A., Inc. Hylar 5000 resin, meeting AAMA 2605 specification.

D. Color: Select from manufacturer's full range of 56 EZ Mix Colors.
PART 3 EXECUTION

3.01
EXAMINATION
A. The installer must examine substrates and conditions under which commercial gutter system will be installed. All wood plates and/or fascia boards shall be installed true, straight, and free of splits, cracks, or other irregularities. Do not proceed with installation until unsatisfactory conditions are corrected.

3.02
PREPARATION

A. Prior to the installation of the commercial gutter system, soffits, extenders, and associated cornice profiles shall be installed.

B. Installer shall thoroughly read and follow manufacturer's installation instructions before proceeding with installation.

3.03
INSTALLATION
A. General: The commercial gutter system shall be installed in strict accordance with manufacturer's printed instructions. Deviations from the instructions are not allowed.

B. Support Brackets: Layout support brackets to provide 1/2" slope in 40 linear feet. Install support brackets with #10 x 2" stainless steel wood screws.

C. Liner: Install concealed gutter liner onto support brackets and fasten to substrates with 1-1/2" aluminum or stainless steel nails. Rivet and seal liner joints with high grade exterior sealant as recommended by gutter manufacturer.

D. Drain & Drain Receiver: Install drain, receiver, flashing clamps and grates at all drain locations. Seal drains to drain receiver with high grade exterior sealant as recommended by gutter manufacturer.

E. Expansion Joints: Install elastomeric expansion joints as shown on plans and/or shop drawings. Maximum expansion joint spacing shall be 40' centers.

F. Install Fascia with concealed splice plates over support brackets and liner. Coordinate and align spacing of joints with associated trims if applicable. Plan spacing of joints so there are no sections of fascia shorter than 48" in length. Check horizontal alignment of fascia during installation and adjust as required. At downspout locations, neatly cut fascia to accommodate downspout.
G. Install interior straps by fully engaging them into liner and fascia, complete by securely riveting.

END OF SECTION 07 71 23

