SECTION 07 71 23
MANUFACTURED GUTTERS AND DOWNSPOUTS
2.05
DRAINWARE

A.  Downspout & Elbows: Provide downspout Model Number DS-FM, in sizes and locations as indicated on plans. Downspouts shall be manufactured from 0.040” aluminum, finished to match gutter fascia mouldings. Downspout elbows shall have heliarc welded joints.

A. Outlets: At all downspout locations provide aluminum outlets to connect liner to downspout.

B. Wall Brackets: Provide Style 1 Wall brackets at 60" maximum spacing (minimum 2 brackets). Brackets shall be manufactured from 0.125" x 1.00" aluminum, finished to match downspout.
