IMPORTANT

These specifications follow the Construction Specifications Institute Master Format but require page formatting to fit your outline.

 * An asterisk means that this portion of the specifications contains size or material options that you can select or change.  If you do not change these specifications then our default options will apply.

** A double asterisk means that this portion of the specifications refers to other divisions of your specifications.  You need to change this information if it is incorrect.
These specifications are to be inserted into your primary gutter specifications.

SECTION 07711

COMMERCIAL GUTTER SYSTEM

2.05
DRAINWARE

  A.  
Downspout & Elbows, provide downspout Model Number

DS-Ornamental Round in sizes and lengths as indicated on plans.

Downspouts shall be manufactured from 0.032" aluminum,

finished to match gutter fascia. Downspout elbows shall

be rolled formed.

  B.   
Outlets, at all downspout locations provide aluminum
outlets to connect liner to downspout.
  *C. 
Wall Brackets, provide Style 1 Wall brackets at 60" maxi-

mum spacing (minimum 2 brackets). Brackets shall be

manufactured from 0.125" x 1.00" extruded aluminum bar,

finished to match downspout.
"16 Ounce Copper is default copper thickness"
